# BCM Door Functionality Validation Report

## Test Execution Summary

Total Test Cases : 14

Passed Test Cases : 9

Failed Test Cases : 3

Not Executes Test Cases: 2

Pass Percentage : 64.29%

Fail Percentage : 21.43%

Not_Executes_test_cases: 14.29%

## Pass / Fail Pie Chart

## Detailed Test Results

| TC_ID | Description | Engine Status | Speed | Expected | Actual | Status | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TC01 | Verify that all vehicle doors are locked successfully when the Lock button on the key fob is pressed | OFF | 0 | Door Locked | Door Unlocked | Fail | Driver Door Open, Rear Left Door Open, Boot Door Open, Lock Failed |
| TC02 | Verify that all vehicle doors are unlocked successfully when the Unlock button on the key fob is pressed | OFF | 0 | Door Unlocked | Door Unlocked | Pass | Driver Door Open, Unlock Successful |
| TC03 | Verify that door unlock operation is allowed using the central unlocking switch when vehicle speed is below 10 km/h | ON | 30 | Door Locked | Door Locked | Pass | Vehicle Speed Above Limit |
| TC04 | Verify that door unlock operation is blocked using the central unlocking switch when vehicle speed exceeds 10 km/h | ON | 9 | Door Unlocked | Door Unlocked | Pass | Unlock Allowed |
| TC05 | Verify that central unlock request is allowed or not at the vehicle speed of 60 km/h | ON | 60 | Door Locked | Door Locked | Pass | Driver Door Open, Rear Left Door Open, Vehicle Speed Above Limit |
| TC06 | Verify that automatic door locking is activated when vehicle speed exceeds 20 km/h | ON | 15 | Door Locked | Door Unlocked | Fail | Speed Below Auto Lock Threshold |
| TC06 | Verify that automatic door locking is activated when vehicle speed exceeds 20 km/h | ON | 25 | Door Locked | Not Executed | Not Executed | One or more mandatory fields are blank. |
| TC07 | Verify that key fob lock request is rejected when the driver door is open | ON | 15 | Door Unlocked | Door Unlocked | Pass | Driver Door Open, Lock Failed |
| TC08 | Verify that all vehicle doors are locked successfully using the central locking switch | OFF | 0 | Door Locked | Door Unlocked | Fail | Driver Door Open, Central Lock Failed |
| TC09 | Verify that all vehicle doors are unlocked successfully using the central unlocking switch | OFF | 0 | Door Unlocked | Door Unlocked | Pass | Rear Right Door Open, Unlock Allowed |
| TC10 | Verify that lock request through the key fob is rejected when the boot door is open and the vehicle is parked | OFF | 0 | Door Unlocked | Door Unlocked | Pass | Boot Door Open, Lock Failed |
| TC11 | Verify lock request when some fields are missing | OFF | 0 | Door Locked | Not Executed | Not Executed | One or more mandatory fields are blank. |
| TC12 | Verify that all vehicle doors are locked successfully when the Lock button on the key fob is pressed | OFF | 0 | Door Locked | Door Locked | Pass | Lock Successful |
| TC13 | Verify that all vehicle doors are unlocked successfully when the Unlock button on the key fob is pressed | OFF | 0 | Door Unlocked | Door Unlocked | Pass | Unlock Successful |

## Failed Test Cases

TC01 : Driver Door Open, Rear Left Door Open, Boot Door Open, Lock Failed

TC06 : Speed Below Auto Lock Threshold

TC08 : Driver Door Open, Central Lock Failed

## Conclusion

A total of 14 test cases were executed.

9 test cases passed.
3 test cases failed.

2 test cases not executed. 

Pass Percentage : 64.29%